| Wireshark 1.5.1 GNS v6.0 Release Notes |  |  |  |
| --- | --- | --- | --- |
| GNS ID | Type | Info | Status |
| 8 | Enh | Added TCP preferences: "Display timestamp values in the summary pane" | Implemented and tested |
| 54 | Enh | Update wireshark.nsi with GNS changes for the install file. | Implemented and tested |
| 66 | Enh | TCP GUI - The following enhancements have been made to the Packet List, "Transmission Control Protocol" tree header, and the TCP tree and subtrees: 1.) When "Transport name resolution" is unchecked, port numbers are no longer redundantly displayed in the TCP tree header: Old TCP tree header: Src Port: 52407 (52407), Dst Port: 8888 (8888), Seq: 101361, Ack: 2761, Len: 1448 New TCP header: Src Port: 52407, Dst Port: 8888, [ACK] [Seq: 101361, Next: 102809 Len: 1448], [Ack: 2761, Win: 65535] 2.) Transmit-related and receive-related info are separately grouped within square brackets in the Packet List and TCP header: [Seq=101961, Next=104857, Len=2896] [Win=135432, Ack=3831] 3.) "Next" is included in the Packet List, TCP tree header, TCP tree: [Seq=99065, Next=101961, Len=2896] [Win=135432, Ack=3831] [Next sequence number: 3831 (relative) 4.) The TCP length (e.g., "Len=2896") is optionally displayed in the Packet List and TCP tree header (e.g., "Len: 2896") 5.) TCP timestamp values (e.g., "TSval=735873 TSecr=452760") are optionally displayed in the Packet List. 6.) Option added to only display "[ACK]" the Packet List and TCP tree header if the SYN, SYN-ACK, ACK of a SYN-ACK, FIN, or RST flags are set. 7.) SACK info has been formatted in a more user-friendly manner and placed immediately after the "ACK=xxxx" in the Packet list. Having the ACk and SACK info in close proximity is more intuitive and useful: Current: Seq=3768 Ack=62177 Win=65535 Len=0 SLE=65073 SRE=65761 New [Seq=273085178, Next=273088074] [Win=135432, Ack=1243774375] | Implemented and tested |
| 67 | Enh | In the TCP GUI, eliminate the "SEQ/ACK analysis" subtree and the items to their associated subtrees: Sequence number Next sequence number Acknowledgement number Window size Options: 'SACK A summary of each of these items is displayed in the subtree header. | Implemented and tested |
| 68 | Enh | More accurate detection of out-of-order frames: In WS v1.5.1 and v1.5.2, out-of-order frames that have not yet been ACKed are labeled as retransmissions. See "out-of-order segments NOT yet ACKed.cap" where frame 7 is out-of-order but labeled a retransmission. Use the IP ID field to resolve this issue. | Implemented and tested |
| 69 | Enh | Configurable RTO periods: RTO periods vary widely among TCP implementations. Those like Celerra DART that adhere to RFC 2988, do not by default allow timeout periods lower than one second but most other implementations such as Linux set the period to as low as 30ms. Allow users to configure the RTO period in TCP preferences and set the default to 201 ms because most implementations the Delayed ACK period is set to a default of 200ms. This value will be used to more accurately detect RTO retransmissions. | Implemented and tested |
| 70 | Enh | The following Recovery Event-related info is accurately determined and displayed: - The six retransmission types: FACK, SACK, Fast, NewReno, RTO and Unwarranted are accurately determined (field names tcp.analysis.retransmission_<fack ¦ sack ¦ fast ¦newreno ¦ rto ¦ unwarranted> - What triggered each retransmission - The recovery target (the highest nextseq when the sender entered recovery and must be ACKed before it can exit Recovery) for each Recovery event in the sender's flow of a connection - The delta time between the the recovery target and its ACK in each Recovery event; and an average those intervals in each Recovery event in the sender's flow of a given connection - The frame number of the original segment that was later retransmitted or if the original is not present, the frame immediately following it. - "[Partner can exit recovery]" in the Packet List and Detail of the frame that ACKs the recovery target [Field "tcp.analysis.can_exit_recovery"] | Implemented and tested |
| 71 | Enh and Bug | Improved support for "Previous segment lost" frames and incorrect original frame number bug has been fixed: When a gap is detected in the byte stream, the packet following that gap should be labeled, "[Previous seg missing]" and the size of the gap should be displayed in the 'Sequence number' subtree: [Gap size: 1460] with field 'tcp.analysis.prev_seg_gap'. If a segment arrives that fills the gap and is designated a retransmission, the label on the first frame should be changed to "[Previous seg lost]" and the frame number of the retransmission should be displayed in that frame's Sequence subtree: [Previous segment(s) were lost] [Retransmitted in frame: 11] Fields: tcp.analysis.prev_seg_lost tcp.analysis.lost_segment_rxmt_at_frame' In the Sequence subtree header of the retransmitted frame, the frame number of the "Previous segment lost" packet is displayed: Sequence number: 2921 (relative), FACK Retransmission (suspected) of frame prior to #4 Orig frame sent prior to frame: 4 Field: 'tcp.analysis.orig_frame'. Note: The current field name, 'tcp.analysis.rto_frame' is misleading because it was used for Fast retransmissions as for RTOs. If a segment that fills the gap is designated as out-of-order, the label in the first frame is changed to "Previous segment out-of-order" and the frame number of the last out-of-order packet in the gap is displayed in that frame's tree: [One or more frames were delivered out-of-order ending at frame: 10951] Filter: 'tcp.analysis.ooo_at_frame' Frames that remain labeled with "Previous segment missing" indicate that the missing frame(s) were actually received but were not successfully stored in the capture file. |  |
|  |  |  | Implemented and tested |
| 72 | Bug | Reordered ACK-only frames and the segments they ACK are now properly labeled: When an ACK-only packet and the segment it acknowledged were reordered, the reordered data-carrying frames were properly labeled as out-of-order but the ACK-only packet was labeled "TCP ACKed lost segment" and the segment it acknowledged was labeled as a retransmission. Test: amazing_stevens_stream-2 frames 2429-2438. If a frame is found to have a gap preceding its seq#, and its nextseq# is less than or equal to the upper boundary of a gap preceding an ACK-only packet (which has the 'ACKed lost segment' flag set), and the IP ID value in that frame is greater than the highest IP ID value seen in that flow; the flag in the ACK-only packet is changed to "ACK-only packet out-of-order" and the data packet is not labeled as a retransmission. If the delta time between the two frames is non-zero, the delta time interval is displayed. | Fixed and tested |
| 74 | Enh | Fixed Congestion Point Analysis (FCPA) is performed based on the average bytes that were in flight when the original segment that was later retransmitted was sent. The average unACKed bytes of original frames is displayed in the 'Sequence number' subtree of each retransmission along with the standard deviation of all the retransmissions: [Congestion point analysis: Avg. bytes in flight when the original frames were sent: 37670; 44 TCP recovery events; Standard Deviation: 121] Field: ' tcp.analysis.avg_ua_of_orig_in_first_rxmt' If the STDEV is reasonably small, the receiver's maximum window size can be set to that value minus that value in order to eliminate or reduce packet loss. One way that average unACKed can be used to mitigate packet loss is to reduce the client's receive window size to just below this amount. | Implemented and tested |
| 75 | Enh | Active SACK blocks, triggers, and gaps are displayed in the Options->SACK subtree: - What triggered the SACK packet to be sent (New SACK block, SACK block update, DSACK, ACK update, or New Data) - The blocks that were included in this frame - The list of active blocks some of which may not have been included in this frame - The gap between each active block and the next. For example: Included SACK blocks: (59281-62177) (65073-65761) Active SACK blocks and gaps (59281-62177) (65073-65761), Total Gaps: 5792 bytes [Triggered by: Block update (165889-181817) (relative)] [Total blocks: 2] ACK (56385) to Block 1 <2896 byte gap> Block 1: 59281-62177 (relative) <2896 byte gap> [Total Gaps: 5792 bytes] Filter fields: tcp.options.sack_trig_<ack ¦ block_update ¦ data ¦ dsack ¦ newblock ¦ unknown> tcp.options.sack_total_blocks tcp.options.sack_block tcp.options.sack_total_gaps | Implemented and tested |
| 76 | Enh | In multi-frame PDUs, the frame number of the last frame of the PDU is now displayed in each segment of the PDU in the Packet List: Earlier versions: [TCP segment of a reassembled PDU] This version: [Full PDU in #1234] In earlier versions, the above message was not displayed in first segment of the PDU. That has been corrected. | Implemented and tested |
| 77 | Enh | Additional TCP window-related info is displayed: The display of Window-related fields was changed from: [Window size value: 16937] [Calculated window size: 135496] [Window size scaling factor: 8] to a subtree with elements: [Window size: 135496 (scaled)] Window size value: 16937 [Window size scale factor: 8 (2**3)] [Scaled window size: 135496] Filter fields: tcp.window_size tcp.window_size_value tcp.window_size_scalefactor tcp.window_scale_known tcp.window_scale_supported | Implemented and tested |
| 78 | Bug | While in TCP recovery, Bytes in Flight (BIF) of retransmitted and new data segments with and without active SACK blocks are now correctly calculated and displayed: With outstanding SACK blocks: When in recovery and SACK blocks are outstanding, bytes in flight should neither include the gap from the ACK to the first SACK block nor the gaps, if any, between the blocks. The bytes in flight value equals the length (seglen) of unACKed retransmissions plus the length of any new un-ACK/SACKed segments transmitted after recovery was entered (snd.fack minus highest_nextseq). NOTE: If a retransmission is itself lost, the retransmission of the retransmitted data should not be included in the BIF value. With no outstanding SACK blocks: BIF equals the highest nextseq sent on this flow minus the highest ACK, minus 1*MSS for each duplicate ACK received, plus the number of bytes in all the retransmissions including those triggered by NewReno with sequence numbers greater than the acknowledgment number. | Fixed and tested |
| 79 | Enh | Field "UnACKed bytes:" (tcp.analysis.unacked) was added to the sequence number subtree: This value is the highest_nextseq minus the highest ACK minus the number of active SACKed bytes that fall above the highest ACK. This field is useful in retransmitted segments where the bytes in flight is often lower (sometimes much lower) than the number of outstanding bytes and serves as an indication of the potential for additional congestion during TCP Recovery. | Implemented and tested |
| 80 | Bug | In some cases a data-carrying packet arrives with the same ACK number and the duplicate ACK counter is set to zero rather than incremented. The fix can be verified in "dup-ack.cap" frames 16 and 18 where the dup-ack in frame 18 is no longer reinitialized. | Fixed and tested |
| 81 | Enh | DSACK is now supported for duplicate SACK ranges per RFC 2883 | Implemented and tested |
| 84 | Enh | Duplicate frame detection: Both data carrying and ack-only packets are detected as duplicated based on the IP ID, TCP flags, length, Seq, Ack, and Window fields. Duplicated frames can be filtered for, Ignored, and optionally the filtered capture saved. New Filters: tcp.analysis.dup_frame tcp.analysis.dup_frame_of | Implemented and tested |
| 91 | Enh | A TCP preference to optionally display the segment length ("Len=XXX") in the Packet List. This is useful in cases where there a tcp.len column already exists and you'd like less space to be taken up in h 'Info' column. | Implemented and tested |
| 213 | Bug | FCPA: The Fixed Congestion Point stat has been made more accurate: Upon entering a Recovery event, the “Target recovery seq# is set to the first byte of the lost segment rather than the last. | Fixed and tested |
| 233 | Bug | Some TCP packets were marked as rexmits. This is occurred when a bogus SACK range was received. This condition was normally caught but in capture, 'bogus-sack-causes-rexmit-storm', it was not and caused all packets from that point on to be marked as retransmissions. This issue has been corrected. | Fixed and tested |
| 236 | Enh | If the 'Display port number in Packet List' preference is set, the port number pair should always be displayed first with a fence following it. TCP analysis messages must be inserted directly after the fence and the existing text must be displayed directly after that message. A new routine, 'col_insert_str_after_fence()' in 'column-utils.c' was created and 'dissect-tcp.c' was updated to utilize this function. | Implemented and tested |
| 237 | Bug | The detection of Reused Port numbers is now working properly. | Fixed and tested |
| 238 | Enh | A new TCP preference has been added to optionally display the TCP flags in the Packet List with following rules: - If checked (TRUE), display all TCP flag types but not for ACK-only packets unless it ACKs a SYN-ACK or FIN. - If UNchecked (FALSE), only display TCP flags if the SYN, FIN, or RST flags are set or it ACKs a SYN-ACK or FIN. - If tcp_analyze_seq = FALSE, display all flag types in the Packet List. - If the "Display TCP summary info in the Packet Details" is enabled, display flags, as well as all the other types of info regardless of other settings, in Packet Details. | Implemented and tested |
| 239 | Bug | Fixed the following in v5.7 and ported it to v6.0: - "Rev UA" (unacked bytes in the reverse flow) now displays the correct value and has effectively obviated the need to display "Tot Gaps" (sum of gaps in SACK packets). - When the relative sequence numbers is unchecked, all sequence numbers (ACK, sle, sre, and "Active SACK block" values) in SACK packets are now displayed as raw rather than relative values. | Fixed and tested |
| 248 | Bug | FCPA: Is only valid if the capture was taken on or near the data sender and must be detected or manually set: The TCP option "Side on which the capture was taken:" is now provided and can be set manually. It is set by default to "Auto Detect" and if set the following criteria are used to determine on which side of the network the capture was taken: (1) If a packet in a one of the flows has a "PACKET LOST" message, the capture was very likely to have been taken on the that flow's side of the network. (2) If the capture has a sufficient number of samples to calculate the average time to ACK a segment in each flow, the smaller of those values will likely belong to the data sender. This method is effective even if the time granularity of the capture exceeds the average ack_rtt. Note: This measure requires that packets with ack_rtt of zero be included in the calculation. (3) If a segment exceeds the MSS, it is an LSO or LRO PDU. If the entire PDU is ACKed, it is an LRO and the capture was taken on the receiver's side. If the capture only contains LSO PDUs, there is no way to detect if it is an LSO or LRO PDU because if a PC connected to a mirror port with LRO enabled was used, the entire LRO will not be ACKed at once. (4) If one of the flows has the TCP_PREV_PACKET_UNSEEN ¦ LOST flag set, the capture was likely taken on the receiver; however, if frames are missing from a sender-side capture it could cause one or both of these flags to be set resulting in a faulty conclusion. If the above measures do not produce a reliable conclusion, a message to that effect should appear in the Sequence number subtree of all the retransmissions in that flow that urges the user to manually set the origin of the capture. In such cases an FCPA will not be conducted unless the origin is specified in the TCP preference. See GNS272 (not yet implemented) for improvements in notifying users of why an FCPA was not performed and the addition of a filter which indicates on which side the side of the network the capture appears to have been taken. | Fixed and tested |
| 262 | Bug | NFS file handle crc32_ccitt hashes have been corrected: tvb_get_string() treats the FH as an ASCII string and thus stops reading at the first zero (0) it encounters. Fix: In dissect_fhandle_data(), g_malloc() allocates 32 bytes, 'tvb_get_string()' was replaced with 'tvb_memdup()', and the malloc's space is freed. | Fixed and tested |
| 263 | Bug | FCPA: In order to improve the accuracy of FCPAs, a Recovery event is not be considered to have ended until all the gaps encountered when recovery began plus any new data that were transmitted before the Recovery event exited are ACKed. This results in some Recovery events not being included in FCPA analyses; however, the results are more accurate. The method currently used to determine the “Target recovery seq#: XXXXXX (XXXXXX bytes remaining)” is based on how TCP is actually supposed to be implemented; therefore, that has been be preserved so that the sender’s reflation of its cwnd following a Recovery event can be examined. | Fixed and tested |
| 264 | Bug | FCPA: The StdDev is calculated based on the unACKed bytes in each event. | Fixed and tested |
| 265 | Bug | Rexmits of missing segs within recovery would sometimes produce errors and has been corrected. | Fixed and tested |
| 266 | Bug | "Original frame could not be determined' errors caused by loss of a segment that comprises an LSO segment has been corrected. | Fixed and tested |
| 267 | n/a | Enter Recovery when 3 dup ACKs are received even if a segment has not been retransmitted. Rejected: The gratuitous ACKs in server-side capture, 'dst_rep--tcp.port-55881.cap' were the result of spurious 'Duplicate of frame' errors in the same capture. When the client in the 'source_rep.cap--port-55881.cap' capture received the spurious dup-acks they were correctly treated as such and did not retransmit the segment; however, it cut the cwnd in half as though it had. This behavior only occurs if SACK has been negotiated but not used. Since this behavior is caused by a bug in the TCP implementation, Wireshark should not be modified. | Not a bug |
| 268 | Bug | Normally the “original frame” is designated as “Original frame sent prior to: xxxx” but in some cases it threw the following error was displayed: Warn Dissector bug, protocol TCP, in packet 2033: tcp_analyze_sequence_number(1234567): The original frame could not be determined In capture ‘rexmits of missing segs within recovery produce errors.cap’, frame 2033 is a retransmission of a segment before frame 553. In frame 2033 there is no “Recovery entered in frame XXX” field so this was not considered as a recovery event. | Fixed and tested |
| 269 | n/a | "The original frame could not be determined" error in capture ‘rexmits of missing segs within recovery produce errors.cap’, threw this error in frame 6499; however, this was caused by a completely different problem than that reported in #268. Frame 6479 was the last segment captured before many SACK packets were received. Since the seq# retransmitted in 6479 was a higher seq# than that of 6479, there was no frame from port 61459 to label “Previous segment missing (or lost)” ergo no way to display an “Original frame sent prior to: XXXX” message and no way to designate that as a recovery event. | Not a bug |

| Known bugs and un-included enhancements |  |  |  |
| --- | --- | --- | --- |
| 247 | ##### Bug | Message "The original frame could not be determined" is inappropriately displayed on half open connections. See capture 'DDC_DM2_FSN0_2.cap' . | Not yet fixed |
| 270 | Bug | Sometimes when an OOO frame belongs after another OOO frame, it is not correctly labeled: See 'bterm1kea-esx04_vmk5_software_testing--port-50264.pcap' A. Frame 52 is an OOO frame that belongs after #44 as stated (no problem here) B. ACK at frame 53 is another OOO that belongs directly after the OOO frame in (A) but is not labeled as such. C. "Unwarranted retransmission" at 54 is really an OOO frame that follows OOO frame #53 thus it should state that it's an OOO that belongs before 44 (or after OOO frame 53). | Not yet fixed |
| 272 | Enh | When an FCPA is not conducted, the reason should be displayed in each retransmission in place of the FCPA statistics line. | Enhancement not yet applied |